PBusDrv.DLL use explanation

PBusDrv.DLL is a 32 digit dynamic link libraries, mostly is used to download PLU data to Pinncale label scale.

Main Data structure and function: (Delphi syntax. For other languages, please refer to use):

1.Data structure

TPlu=record			//PLU data structure

Name:         PChar; 	//product name, 36 characters

LFCode:       Longint;	//Lfcode,1-999999,only way to identify each kind of product

Code:         PChar;	//product code，10 digit, used to form barcode

BarCode:      Longint;	//barcode type,0-99

UnitPrice:    Longint;	//unit price, did not have decimal mode, 0-9999999

WeightUnit:   Longint;	//weight unit 0-12

Deptment:     Longint;	//department,2 number, used to form barcode

Tare:         Double;	//tare weight, it should within 15Kg after logic conversion

ShlefTime:    Longint;	//shelf time,0-365

PackageType:  Longint;	//package type,0:normal/ limitative weight mode,1:weight fixed,

//2:price fixed,3:barcode printer mode

PackageWeight:Double;	//package weight/limitative weight, it should within 15Kg after logic conversion

Tolerance:    Longint;	//package error tolerance,0-20

Message1:     Byte;	//message 1,0-197

Reserved:     Byte;	//reserved

Reserved1:    Word;	//reserved

Message2:     Byte;	//message 2,0-197

Reserved2:    Byte;	//reserved

MultiLabel:   Byte;	//multi-label,0-255,8 Bit corresponding to A0-D1 respectively

Rebate:       ShortInt;	//rebate,0-99

Account:      LongInt;	//enter an item in an account, reserved

end;

PPLU=^TPLU;

TPLUCluster=Array[0..3]of TPLU;	//one PLU cluster including 4 pcs PLU

PPLUCluster=^TPLUCluster;

THotkeyTable=Array[0..83]of LongInt;	//each array element is a PLU lfcode,

//84 pcs element corresponding to 84 pcs PLU

PHotkeyTable=^THotkeyTable;

For detailed explanation, please refer to user manual’s program interface part.

2.Function(suppose label scale sw (LBMNC) is installed under XX directory)

Connect function

Function PBusConnect(RefLFZKFileName:PChar;//lf font list file name,including path,

//usually point to XX\bin\lfzk.dat

//if used character set，point to XX\bin\lfzkfull.dat

RefCFGFileName:PChar;	//configuration file name, including path, usually point to //XX\bin\system.cfg

SerialNO:DWord;		//label scale serial number, if is Ethernet, please transfer IP address as

//DWord，such as 192.168.0.1 transfer to $C0A80001

CommName:PChar;		//equipment name, such as:COM1,COM2，if is Ethernet, it is:IP

BaudRate:Integer	//communication speed, set as 0

):Integer;far; external 'PbusDrv.dll';

Function PBusConnectEx(RefLFZKFileName:PChar;// lf font list file name, including path,

//usually point to XX\bin\lfzk.dat

//if use character set，point to XX\bin\lfzkfull.dat

RefCFGFileName:PChar;	//configuration file name, including path, usually point to //XX\bin\system.cfg

IPAddr: PChar;		//label scale IP address, such as:192.168.0.1

):Integer;far; external 'PbusDrv.dll';

Disconnect function

Function PBusDisConnect(SerialNO: DWord	//label scale serial no.

):Integer;far; external 'PbusDrv.dll';

Function PBusDisConnectEx(IPAddr: PChar;	//label scale IP address，such as:192.168.0.1

):Integer;far; external 'PbusDrv.dll';

send PLU function

Function PBusTransferPLUCluster(PPC:PPLUCluster	//PLU cluster

):Integer;far; external 'PbusDrv.dll';

send Hotkey function

Function PBusTransferHotkey(HotkeyTable:PHotkeyTable; //hotkey list

TableIndex:Integer	//hotkey list no., total two list,0 and 1

):Integer;far; external 'PbusDrv.dll';

send message function

Function PBusTransferMessage(Id:Integer; //info code，1~197

PMessage:PChar; //information text

DataLen:Integer //information text length

):Integer;far; external 'PBusDrv.DLL';

transform PLU to character string

Function PBusPLUToStr(LPPLU:PPLU;		//PLU data

LPStr:PChar		//character string(one line of TXP file), this finger must distribute space.

):Integer;Stdcall;far; external 'PbusDrv.dll';

transform character string to PLU

Function PBusStrToPLU(LPStr:PChar;		//character string(one line to TXP file)

LPPLU:PPLU		//PLU data

):Integer;Stdcall;far; external 'PbusDrv.dll';

clear all PLU

Function PBusClearPLUData:Integer;Stdcall;far; external 'PBusDrv.dll';

3.Flow

(1)	PBusConnect	if returned value<0, jump out

|

(2)	PBusTransferPLUCluster/PBusTransferHotkey	if returned value<0, jump to (4)

|

(3)	PBusTransferPLUCluster/PBusTransferHotkey	if returned value<0, jump to (4)

|

.	if returned value<0, jump to (4)

.

.

(4) 	PBusDisConnect

4.Examples

Example under Delphi60 catalog is writen with Delphi, could pass test under Delphi6.0.  Example under VB60 catalog is written with Visual Basic, could pass test under VB6.0.

Example under VC60 catalog is written with Visual C++, could pass test under VC6.0.

Example under PB65 catalog is written with PowerBuilder, could pass test under PB6.5.

Remarks：

1．Explanation of lf font bank and system parameter

PbusConnect connect function cites two file: lfzk.dat and system.cfg, lfzk.dat is lf font bank character list，store some system setting in system.cfg . We intensively suggest to cite corresponding file under link32 install catalog，to make sure that when Link32 sw modifying system parameter, it could reflect to dynamic link libraries in time。For example：There is a option “lf common font bank usage character set” of link32 system option，after modification of it， this parameter setting will be stored in system.cfg，if PbusConnect connect function cites system.cfg of link32 setting catalog ，the dynamic link libraries will download using new configure, or it will confuse the font bank. Of course, you could place these two file on other places, but must make sure update these two file in time and in-phase.

2．Setting of data align mode of delphi 6 and Delphi 7.

Set delphi6 and delphi7 as below,

Project->Options->Code generation->Record field alignment select 4

If do not want to change default setting (default value is 8) of Delphi data align mode or use Delphi5，could use PBusDrv.DLL of other catalog.